# RISC-V emulator

## Workflow of the CPU:

- Fetch instruction from memory

- Decode instruction

- Execute instruction

- Increase program counter

- Go back to step 1.

## Important details of RISC-V 64 bits:

- 64 general-purpose register (from 0 to 31)

  - 0th register always contains a zero

  - each register has the size of 64 bits

- PC (program counter)

  - counts which instruction are we at

  - doesn’t stored in the general-purpose registers

- Instructions has the size of 32 bits

## Instruction formats:

- Six basic instruction formats:

  - R-type for register-register operations

  - I-type for immediate and load operations

  - S-type for store operations

  - B-type for conditional branch operations (the full immediate must be reconstructed)

  - U-type for long immediate instructions

  - J-type for unconditional jumps

- Notations:

  - opcode: Code of the exact operation (eg: plus)

  - rd: Destination register

  - rs1/2: Source register 1/2

  - funct3/7: Sub functions of the operations defined in opcode

  - imm: immediate value (basically hard coded integers)

## Instructions needed to be implemented:

- li  Load Immediate (I-Type)

  - It is actually calling addi (what is an I-Type) on machine code level

  - loading an integer into a register

- mulw  MULtiply Word (R-Type, funct3)

  - only works on 64 bit versions

  - takes the lower 32 bits as signed of r1 and r2 and multiplies them into the rd 64 bit register

- beqz  Jumps when register is zero (because it’s a more specific version of beq it is B-Type)

  - It check if r1 is zero, if yes sets the pc to the immediate value (jumps)

- add  adds two register together (R-type)

- addi  add immediate to register value (I-type)

- j  jumps to label (what is basically an immediate) (J-type)

- jal  jump and link (J-type)

  - basically like calling a function.

  - it saves to the current program counter in rd, then sets the pc to the immediate value

  - if we want to go back to where jal is called we just jump to the value stored in rd

- lw  Loads 32 bit value into rd (I-Type)

- sw  Stores lower 32 bit of rs2 into r1 (S-Type)

- ret  Return (I-Type)

  - Returns to the ra value which was saved by the jal